# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: “Supply of special purpose equipment- Fire

- vehicle for all terrain with supporting equipment and

- Collapsible fire pump and IT equipment”

Publication reference:  CB007.1.31.189 – TD05

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Note: The selected tenderer is obliged to submit all the documentation for the vehicle registration in accordance with the current legal regulations of the Republic of Serbia

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered |  | 4. Notes, remarks, ref to Documentation | 5. Evaluation Committee’s Notes |
| --- | --- | --- | --- | --- | --- | --- |
| LOT 1 |  |  |  |  |  |  |
| 1 | Fire vehicle for all-terrain with supporting equipment | 1pcs |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |
|  |  |  | Origin: |  |  |  |
|  | Minimum technical requirements or equivalent |  |  |  |  |  |
|  | Product/type, model: Type Firefighting & Rescue vehicle suitable for forest fires, outdoor fires and fires in hardly accessible and rural terrain, for 3 firefighters rescuers (1+2) In accordance with SRPS EN 1846-2, category 3 Engine Turbo charged diesel, minimum power 230kW, EURO 6 Power transmission Two axle drive 4 X 4 Front, rear and interaxle differential lock Power outputs PTO for the pump Dimensions Wheelbase max 3700mm Length max 7500mm Width max 2600mm (without rear mirrors) Height of the vehicle/superstructure max 3300 mm Clearance min 400 mm GVW 15 tons Fordability min 850mm tilting angle : min 25° Brakes Air brakes, ABS Steering system Servo Cab (Doors, Seating capacity, other equipment) Number of seats 1+2 Driver seat adjustable Number of doors 2 Access and entry in accordance to SRPS EN 1842-2 cat. 3 PTO pump switch LED light on vehicle switch Light switch for the rear (pump) storage box Self-protection switch Signal lamp for storage door open Signal lamp for self-protection Installation for radio communication device (power and antenna wires) Visual and acoustic signalization Two blue rotating lights or blue blinking light consoles on the front part positioned on the top of the cabin and one light on the top of the rear part of the superstructure. Lights should be positioned in a manner that they do not interfere with regular firefighter work. All lights should be protected with steel protection net. Acoustic signalization with at least two different horns and a loudspeaker of 100W. Electrical installation Voltage 24V Batteries in accordance with SRPS EN 1846 Alternator 28V, ≥70 A – 1960 W Fuel tank Volume : min. 180l or 4 hours of power for the equipment powered by the vehicle engine or 300km of driving respectively Winch for self-extraction Mounted in the front Electrically powered Pulling force min. 5.000 kg Superstructure Material and Construction: Monoblock superstructure is made of stainless steel AISI 316L The superstructure to be connected to the chassis trough a sub frame. The superstructure should be connected to the sub frame through one rocket and stainless steel mesh shock absorbers. The sub frame is connected rigidly to the chassis. 3.000l water tank placed on the factory recommended support built on chassis. Minimum 1 storage on each side and 1 storage in the back. All storages are equipped with internal LED light and aluminum roll doors for protection from outside influences and with outside locking. First storage from the truck cabin on the right side is equipped with sliding SCBA carrier for easy access and utilization of equipment. Top of the superstructure is covered with anti-slip material and it can be reached with ladder mounted on the superstructure in accordance with SRPS EN 1846-2. Roof is equipped with at least one aluminum tool box. Dimensions of the box should be suitable for hand tools required in additional equipment list. Spare wheel and small manual hoist is also mounted on the roof of the superstructure. All wires protected against heat. Water tank Stainless steel AISI 316L water tank with capacity of 3.000l with inside separation for prevention of sudden water movement. Tank opening on the top of the tank with possibility of opening from the top of the superstructure. Filing inlet one Ø75 mm Drainage outlet Ø 52mm on the lowest part of the tank with ball valve. Tank vent: vent pipe on the tank Foam tank Foam tank made of Stainless steel AISI 316L with capacity of 200l. Foam tank should have one opening on the top of the tank with possibility of opening from the superstructure, with under pressure valve and foam level indicator. One outlet connecting to the pump mixer. One outlet for foam discharge Pump Stationary centrifugal pump with two stages, enclosure made of brass or bronze with stainless steel shaft. Low pressure impeller & high pressure turbine made of brass or bronze. Piping made of stainless steel AISI 316L with high anti corrosion protection. The pump is positioned in the back of the superstructure – medium pressure 3.000l/min at 8 bar and high pressure 250l/min at 40 bar. One inlet A Ø110 mm for supplying from external source. Two B Ø75 mm Storz outlets One outlet for medium pressure hose reel One outlet for high pressure hose reel Manual or electrical medium pressure hose reel with 30m reinforced rubber hose according to SRPS EN 1947 Ø45mm with pistol type nozzle. Installed foam mixing system after the pump with mixing range from 1%-6% Manual or electrical high pressure hose reel with 30m reinforced rubber hose according to SRPS EN 1947 Ø25 mm with pistol type nozzle Installed foam mixing system after the pump with mixing range from 1%-6% Pump control panel with medium and high pressure gauge, vacuum meter, operating hours counter, manual controls for medium and high pressure reels, vacuum device and foam mixing systems for medium and high pressure. Self-protection Anti Roll Over Protection Bar with water nozzles for thermal protection of the cabin. All wheels are protected with water nozzles and water supply is provided by independent Electrical pump (Possibility of operating while main power source is not operating and while vehicle is standing or in motion. Superstructure stored equipment (additional equipment) SCBA with composite cylinder with volume of min. 6l on 300bar in accordance with SRPS EN 137 type 2 for professional firefighters and with additional spare composite cylinder – 2 pieces Suction hose Ø110 mm Storz , length 1,6m – 4 pieces Suction basket made of light alloys with non return valve and Storz coupling A – 1 piece Static rope 30m Ø10mm – Ø11mm – 1 piece Short rope with carabiner and loop, length from 1,2 to 2m in accordance with SRPS EN 354 – 2 pieces Universal wrench for ABC Storz couplings – 2 pieces Collecting breeching B/CC Storz, made of light alloys with clappered inlets – 1 piece Dividing breeching (splitter) B/CBC Storz, made of light alloy with ball valve – 1 piece Dividing breeching (splitter) C/DD Storz, made of light alloys with ball valve – 6 piece Adapter coupling C/D Storz – 3 pieces Delivery hoses B, length 15m – 3 pieces Delivery hoses C, length 15m – 6 pieces Delivery hoses D, length 20m – 25 pieces Nozzle D Storz made of light alloys with variable water patterns – 6 pieces Heavy foam branch pipe 200l/min – 1 piece Medium foam branch pipe 200l/min – 1 piece Foam inter-mixer 200l/min C Storz – 1 piece Water nozzle C Storz, with turbine and flow rate control 100-400l/min, with variable water patterns – 1 piece Hammer-Axe with 450mm handle (in roof box) – 1 piece Crowbar (in roof box) – 1 piece Pickaxe (in roof box) – 1 piece Fire extinguisher S9 – 2 pieces Fire broom with handle to max. 2000m and weight of max 3kg (in roof box) – 4 pieces Shovel (in roof box) – 2 pieces Hoe (in roof box) – 3 pieces First aid kit – 1 piece |  |  |  |  |  |
| 2 | Collapsible fire pump 50 pcs |  |  |  |  |  |
|  | Manufacturer’s name |  |  |  |  |  |
|  |  |  | Origin: |  |  |  |
|  | Minimum technical requirements or equivalent |  |  |  |  |  |
|  | Product/type, model: Backpack is made of rubberized canvas or other watertight materials with similar characteristics or Cordura with flexible inner water tank. Backpack is connected to manual piston pump by the rubber hose. Backpack is equipped with carrying handle and shoulder straps and waist straps for better weight distribution. Backpack is equipped with filling opening with sealing cap. Backpack volume: max 25l Stream pattern: full jet and spread Stream range: full jet min 10m, spread min 5m 2 year warranty for manual piston pump and nozzle |  |  |  |  |  |
| LOT 2 |  |  |  |  |  |  |
| 3 | Desktop computers – 5 pcs Minimum technical requirements or equivalent Processor Intel Core i3 Motherboard PCIe/DDR3/SATA3/USB3.0/LAN/7.1 RAM 4GB DDR3 HDD 500GB SATA2 Graphic card Integrated Optical device DVD RW Monitor Panel size: 19.5'' Recommended Resolution: 1366x768 Brightness: 200cd/m2 Contrast Ratio: 600:1 Response Time: 5ms Input Signal: Analog D-Sub Viewing Angle (H/V): 90/50 Factory warranty: 3 years Color: black Keyboard YU set USB-PS2, black Mouse Optical scroll, USB-PS2, black Desktop case with supply Desktop, Power supply min 450W ATX Operating system Windows 10 |  |  |  |  |  |
| 4 | Laser Printer – 3 pcs Minimum technical requirements or equivalent Paper dimensions A4 Resolution 600x600 dpi Printing speed min 35 ppm A4 Maximum Monthly Duty Cycle min 50000 Interface USB 2.0; Ethernet 10/100 Time to First Page < 9 sec Paper Input Capacity 250 Manual paper input yes Duplex (2-sided) Printing Integrated Duplex Supplies Yield min 3,500-page Toner Cartridge Printer Languages PCL 6, Post Script Microsoft Windows Operating Systems Supported Windows XP, Windows 7, Windows 8.1, Windows 10 |  |  |  |  |  |
| 5 | Tablet – 1 pcs Minimum technical requirements or equivalent Display 10.1" (1920x1200), LED backlight, 5- point capacitive touch, water rejection, palm rejection, gorilla glass technology CPU Intel Bay Trail-I E3827 RAM min 2GB DDR3 Storage min SSD 80GB Webcam 2 Mpix (front) 5 Mpix(rear) Wireless WLAN 802.11ac/a/b/g/n Connection GNSS (GPS/Glonass/BeiDou) WWAN 3.5 or 4G LTE Housing Rugged plastic Power supply standard battery 4500mAh, AC-adapter 100-240V, backup battery 2500mAh - battery pack built-in for hot swapping Sensors gyro, accelerometer, ecompass, ambient light sensor Interfaces USB3.0, USB2.0, RJ45, RS232, audio, HDMI, micro SIM, DC-in, pass through connector Certification IP65, MIL-STD 810G OS Android 4.4 |  |  |  |  |  |
| 6 | External hard disc-2 pcs Minimum technical requirements or equivalent Capacity 2TB Interface USB 3.0 Format 2.5" Microsoft Windows Operating Systems Supported Windows XP, Windows 7, Windows 8.1, Windows 10 |  |  |  |  |  |
| 7 | Laptop computers-3pcs Minimum technical requirements or equivalent Processor Intel Core i3 Motherboard Giga LAN, WLAN 802.11b/g/n, D-sub and HDMI out, USB 2.0, USB 3.0, SD MMC card reader, 1.3 MP Webcam, microphone RAM 4GB DDR3 HDD 500GB SATA2 Graphic card Integrated Optical device DVD RW Monitor Panel size: 15,6'' Recommended Resolution: 1366x768 Keyboard YU set keyboard, numeric keyboard Mouse touchpad Operating system Windows 10 Battery 4-cell Li-ion AC/DC Adapter yes Colour Black Warranty 3 years |  |  |  |  |  |