CORRIGENDUM

CONTRACT NOTICE No 03

II.1.1) Contract Notice Title: Construction of an artificial grass football field

II.1.1) Contract Notice Reference Number:  CB007.2.21.072 –PP1-TD01

VII. 1.1) Reason for change

Modification of original information submitted by the contracting authority.

VII.1.2) Text to be corrected in the original notice

The following alternations and/or corrections are made

In the document  VOLUME 3 Technical qualifications  - Construction of an artificial grass football field architectural and construction part I

/ Position 13 /4.3.

Instead of  Supply, transport and installation of synthetic grass. Purpose: football. The required characteristics of artificial grass:

* Fiber type: Polyethylene (PE) -monofilament  avke

* Fiber fineness: min. 15. 000 dtex

* Fiber thickness: min. 410 microns

* Number of loops / m²: min. 9.449

* Number of fibers / m²: min. 113.386

* Artificial grass color: green - two shades

* Primary substrate: Polypropylene (PP) 100%* Secondary substrate: SBR latex

* Grass tear resistance (N): ˃30

* UV stabilization (QUV-lamp A) (h) : 3.000

* Color stability-gray scale: ˃4

* Fiber height: min. 60mm

* Total weight g / m²: min. 3090

* Terrain marking according to the project with white lines 12 cm wide

* Filling: quartz sand and SBR rubber granulate, all according to the manufacturer's specification

Mandatory conditions:
* FIFA recommended manufacturer
* Technical specification of grass
* FIFA LABO TEST with Fifa Quality & Quality pro for artificial grass
* LABO TEST EN Standard 15330-1
* LABO TEST synthetic grass fibers
* Warranty on synthetic grass for large terrains: 8 years
The calculation is performed per m² of placed synthetic grass.
105x70 = 7350 m2

Read: Supply, transport and installation of synthetic grass. Purpose: football. The required characteristics of artificial grass:

* Fiber type: Polyethylene (PE) -monofilament  avke

* Fiber fineness: min. 15. 000 dtex

* Fiber thickness: min. 410 microns

* Number of loops / m²: min. 9.449

* Number of fibers / m²: min. 113.386

* Artificial grass color: green - two shades

* Primary substrate: Polypropylene (PP) 100%* Secondary substrate: SBR latex

* Grass tear resistance (N): ˃30

* UV stabilization (QUV-lamp A) (h) : 3.000

* Color stability-gray scale: ˃4

* Fiber height: min. 60mm

* Total weight g / m²: min. 3090

* Terrain marking according to the project with white lines 12 cm wide

* Filling: quartz sand and SBR rubber granulate, all according to the manufacturer's specification

Mandatory conditions:
* FIFA recommended manufacturer
* Technical specification of grass
* FIFA LABO TEST with Fifa Quality & Quality pro for artificial grass
* LABO TEST EN Standard 15330-1
* LABO TEST synthetic grass fibers
* Warranty on synthetic grass for large terrains: 8 years
The calculation is performed per m² of placed synthetic grass.
105x70 = 7350 m2

NOTE: All conditions specified within technical specification/required characteristics of artificial grass should be provided certified with the offered product and that no additional tests are required.

If the characteristics related to Labo test EN standard 15330-1 and Labo test for synthetic grass fibers are already included in FIFA LABO TEST tenderer should  present  appropriate clarification which proves tenderer's  statement.

VII.2) Other additional information:

All other terms and conditions of the contract notice, and tender dossier remain unchanged. The above alterations and/or corrections to the contract notice are integral part of the contract notice and tender dossier.