# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment

Publication reference: CB007.1.31.364-PP2-TD3

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Lot 1 - Truck Tipper: - Gross weight (GVW) min. 26000kg - Wheelbase distance min. 3000mm - Drive wheel propulsion min. 6x4 - Multi use tires 10 + 1 spare, min. R22.5 - Suspension: front and rear axles with mechanical suspension - Steering front axle, load capacity min. 8000kg - Rear tandem axles with hub reduction, load capacity min. 21000kg - Rear differential lock - Steering system: power steering - Diesel engine, displacement min. 7500ccm3, min. 200kW, torque min. 1400Nm, min. Euro 4 - Synchronized manual transmission - PTO output in line with the tipper hydraulic, cab control - Fuel tank min. 300 lit. from steel - Min. 50 lit. urea tank - Brake system - Front: disk; Rear: disk or drum brakes - Radiator guard - Alternator min. 80A - Reverse buzzer - Security system: ABS + ASR with control command for the off road mode - Cruise control - Cab painted in white color - Cab type: day cab, min. 1+1 seat - Min. 1 rotating light on the cab roof - External sun visor - Pneumatically supported driver’s seat with armrest - Seat belts and headrests on seats | < fill offered specification> | < fill or leave blank> |  |
| 1 | - Left and right rear view mirrors each with a wide angle mirror with heated and electro- adjustable - Electric front windows - Air conditioning in the cabin - Front fog lights - Mechanical protection on front light groups - Front steel bumper - Tiltable rear bumper - Digital tachograph - Trip computer - FMS data interface - Coded key, immobilizer, min. 2 pieces - Basic toolkit with jack, triangle, portable flashlight, tyre inflation hose Tipper box: - Capable of tipping on three sides - Volume of cargo crate with capacity of min. 10m3 - Body floor from steel with abrasion resistance, thickness min. 6mm - Boards from steel with abrasion resistance, thickness min. 4mm - Single piece sideboards - Two-way opening side boards - Front board with roof cab protection - Rear gate with automatic opening device - Central telescopic cylinder - Pneumatic control from the cab - PTO and gear pump - Spare wheel with holder - Lateral protections - Rear reflecting plates - Painted in white color Warranty period min. 1 year |  |  |  |
| 2 | Lot 2 - Backhoe Loader - Engine power min. 48 kW - 4 (four) wheels drive - Rigid chassis machine with parallel loader arm kinematics - Operation weight min. 9400kg - Engine emission level acc. to TIER IV final standard - Engine displacement min. 3.300 ccm3, max. 4500 ccm3 - Powershift transmission - Four wheels steering - Hydraulic oil pressure max. 260bar - Hydraulic pump flow rate minimum 155 l/min - With Pipework for hydraulic hammer - Front loading hydraulic spoon min. volume of 0,8m3 - Max. depth of digging 8000mm - Movable backhoe device on horizontal axis: left and right - Reach of excavator hand max. 9500mm - Digging bucket width min. 600mm - Pneumatics: front min. R28 and rear min. R28 - Working light: front min. 2 pcs and rear min. 2 pcs - Rotation beacon on roof - With all additional equipment that is delivered with the machine Warranty period min. 1 year |  |  |  |
| 3 | Lot 3 - Mini Excavator with a Hydraulic Hammer - Engine power min. 18 kW - Propulsion on continuous track - Rubber tracks - Depth of digging max. 4500mm - Max. horizontal reach 6500mm - Machine weight min. 4000kg - Hydraulic hammer appropriate to the machine - Hydraulic leaning system of whole upper structure of the machine - ROPS/FOPS cabin with heating - One digging bucket min. 400mm - One digging bucket min. 600mm - Ditching bucket min. 1200mm Warranty period min. 1 year |  |  |  |