# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment 	p 1 /…

Publication reference : CB007.1.31.364/Boljevac/T3

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

All functional and technical specifications are to be considered minimum and mandatory for all equipment and are cumulative, unless otherwise specified. A single product must comply with all specifications listed in its category/section simultaneously, to be considered compliant. All products must have at least 1 (one) year warranty if not specified otherwise. Refurbished, used or pre-owned components, devices or equipment are not accepted. For all components, products with either equivalent or exceeding specifications are accepted, but bidders must provide valid proof for each claim. The bidder must ensure that by including products and services equivalent or exceeding the specifications do not break the functionality and / or the integrity of the final solution.

Memorandum of the Tenderer must contain description of the technical characteristics related to offered item, which must at least contain technical characteristics requested by the Contracting Authority, signed by the authorized person and certified by stamp.

List of authorized services for offered items in Republic of Serbia, which is certified by the manufacturer of the offered goods, with addresses and contact phones ( for chassis and truck upgrade)

Description of the technical characteristics, the manufacturer's certificate, the authorization and the list of authorized services must be original and signed by the responsible person of the manufacturer and certified with the stamp of the manufacturer. If the said documentation is provided in a foreign language, it must be translated into Serbian and verified by a court interpreter. The offered chassis and truck upgrade must fulfil all the requirements stipulated in the technical specification from the tender documentation, otherwise the offer will be rejected as unacceptable.

If the tenderer is simultaneously the manufacturer of the chassis or truck upgrade that is offered, the Authorization referred to in point 3. delivers in the form of Statements under material and criminal responsibility that he is the manufacturer of the offered chassis / upgrade and signed by an authorized person and stamped.

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| I | LOT 1 |  |  |  |
|  | Truck tipper - one piece Permissible front axle max load: 8000 kg Permissible rear axle max load: 26000 kg MOTOR, COOLING SYSTEM, BRAKE Protection against dust extraction from the fan EDC - engine regulation Fuel filter Pre-filter for fuel with heater and water separation Heating fuel filter Connection for external rotary speed control (ZDR) 1-cylinder compressor, 360 cc Reinforced Motor Brake Insect protection net in front of the radiator Electronic speed limitation at 89km / h +1 km / h of tolerance Cruise control EC type-approval certificate Declaration of the manufacturer Noise level 80 dB (92 / 97EEC) The suction of the air is raised with a dry air filter Exhaust pipe down to the left Manual transmission (16 forward + 2 back) The front axle VO-09 is right The front movements are parabolic 8,0t Medium-high clearance Front axle stabilizer Permitted front axle load 8000kg The last movable parabolic 13,0t Rear axle drive HUB rear axle HPD-1382 / HP-1352 - with side reducers Differential lock on the rear axle Rear axle suspension, mounted up Stabilizer on both rear axles Permissible rear axle load 2x13000kg |  |  |  |
|  | TRANSMISSION RATIO Portable relay ratio i = 3.63 WHEELS Steel wheels with 10 holes 9.00-22.5 on the front axle 1 Steel wheels with 10 holes 9.00-22.5 on the rear axle 1 Steel wheels with 10 holes 9.00-22.5 on the rear axle 2 Steel wheels with 10 holes 9.00-22.5 at the spare wheel SPARE TIRE WITHOUT spare wheel holder RESERVOIR Reservoir 300l steel, right, AdBlue 35l Mechanical protection under the reservoir Sieve in the fuel tank opening Lock of the tank with lock AdBlue tank lock with lock MANAGER Adjustable height and depth control Lock the steering wheel with the immobilizer CHASSIS Rear underbody protection, round Side protection from underfloor WITHOUT front protection against underrun Three-piece steel bumper Rear transversal chassis carrier with trailer holes Preparation (Chassis connection) ELECTRICAL SYSTEM Two-way electric siren ABS socket for trailer at the end of the chassis Socket for trailer 24V 7 + 7-pin at the end of the chassis Sockets in the cabin 24V and 12V Mechanical circuit breaker 2 batteries 12V 175Ah Battery box up to 175 Ah "Maintenance-free" batteries Alternator 28V 120A 3360 W Longlife Eco |  |  |  |
| II | LOT 2 |  |  |  |
| 1. | Backhoe loader - one piece turbo-diesel engine, Rigid chassis with axles for severe working conditions; Transmission hydrodynamic ; Synchronized four-speed manual transmission Steering system - SRS; Front loading hydraulic opening "6 in 1" bucket, volume min 0.9 m3, loading height min 3.050 mm), for bathing, loading, basing, straightening, filling back and pallet filling; Telescopic backhoe; Cabin with heating; Overall machine length max 5,900 mm; Overall width of the machine max 2,700 mm; Transport height of the machine max 3.700 mm; Maximum digging depth 6.000 mm; Hand held on the ground max 6.720 mm; Hydraulic stabilizers with adjustment; |  |  |  |
|  | Automatic leveling of the bucket; Rotary light; Depth seat; Noise in the cab max 75 dBa; Fuel tank capacity min 140 l; Hydraulic hammer installation ; Digging bucket of 600 mm wide ; |  |  |  |
|  | Mini excavator- one piece Water cooled 3-cylinder diesel engine Operating weight of the machine max 2.900 kg; Machine width max 1.600 mm; Transport height max 2.500 mm; Conveyor length max 4.200 mm; booths with heating; Dipper with a length of max 1.200 mm; Max digging depth 2.900 mm; Hand reach on the ground max 4.800mm; Hydraulic installation for the hammer on the rowing arm; With rubber tracks, 250 mm wide; Seat belt ; Rotary light; Work lights; Dozer board max 1.600 mm in width, 350 mm in height; 460 mm wide bucket; Hydraulic hammer |  |  |  |