# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of equipment for Surdulica Cultural Center (SCC)

p 1 /…

Publication reference: 2014TC16I5CB007-2015-1-CB007.1.11.273 – 4- PP2-SUPPLY (404-313/17-01)

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | The 2-way loudspeaker system. Quantity 6. (Sceen monitoring) The two-way full range loudspeaker systems shall incorporate an 8" cone transducers and a 1" exit compression drive on dispersion horn, with a nominal impedance of 8 Ohm. The system's horizontal coverage shall range from min 50º to min100º, with vertical coverage of min 55º. The horn shall be rotating in 4 directions. The system shall produce a Sound Pressure Level (SPL) of min 98 dB SPL on axis at 1 meter with a power input of 1 Watt, and shall be capable of producing a peak output of min 132 SPL on axis at 1 meter. Frequency response shall be min 60Hz to max 20kHz (±3dB). Electrical connections shall be made via two 4-pol speaker twist connectors. The loudspeaker enclosure shall be trapezoidal in shape. It shall be constructed of 1/2-in thickness Baltic birch plywood with textured black coating and it shall be finished in black. The front of the loudspeaker shall be covered with a perforated steel grille. |  |  |  |
| 2 | Controller device. Quantity 3. (Sceen monitoring) An advanced analogue processor shall configure from a combination of min two electronically balanced analogue inputs and min three outputs with each output having facilities for crossover, parametric EQ, mid-filter limiters, and delays, in a single rack-mountable unit. The controller device shall provide dynamic, voice coil temperature and diaphragm displacement protection in order to prevent transient audio peaks from causing distortion further up the audio chain or damaging loudspeaker components. Maximum input shall be 22dBu. Total harmonic distortion: less than 0.1%. The device shall have clear front panel LED indication. The chassis shall be constructed of aluminum profile + sheet metal material, suitable for the peripheral devices. Input power shall be 110-220VAC, 50-60Hz with a rear panel earth-lift. |  |  |  |
| 3 | Power amplifier. Quantity 3. (Sceen monitoring) Power amplifier with: Output power, 1kHz: 750W on 8ohm,1 100W on 4ohm, 2200W on 2ohm. Cooling: 2x variable-speed fan. Power band width: 10 Hz – 40 kHz Total harmonic distortion: less than 0.1% Frequency response: ±0.5 dB, f = 20 Hz – 50 kH Power amplifier shall be equipped with independent sweepable high-pass and low-pass filters on each channel. Also, equipped with speaker twist connectors jacks, binding post, and 1/4-inch output jacks for each channel, balanced XLR and 1/4-inch TRS jacks for channel input. Amplifier shall be in a single rack-mountable unit, constructed of aluminum profile + sheet metal material, suitable for the peripheral devices. |  |  |  |
| 4 | Flight case 16U. Quantity 1. Type: Removable front and rear. Material: min 7 mm plywood multilayered glued, black laminated, with 1" layer of shock-absorbing foam between rails and outer wall. Rack shall be with heavy duty fittings, metal railing/ball corners, aluminum profile frames with rounded edges, spring loaded carry handles and wheels. Also, the rack shall be mounted with handles for transport, rails and screws for installation of rack components. Rack shall strip front and rear. All washers + nuts + screws shall be included. |  |  |  |
| 5 | Full-range loudspeaker unit for stand-alone. Quantity 9. (Main front speaker systems) Full-range loudspeaker unit for stand-alone, curved array or line array application shall incorporate high-power min 6.5" LF driver and a 1" throat drive, with a nominal impedance of 8 Ohm. The system's horizontal coverage shall range from min 80º, with vertical coverage of min 20º, 0° to 20° splay when arrayed. The system shall produce a Sound Pressure Level (SPL) of min 95 dB SPL on axis at 1 meter with a power input of 1 Watt, and shall be capable of producing a peak output of min 127 SPL on axis at 1 meter. Frequency response shall be min 80Hz to max 20kHz (±3dB). Electrical connections shall be made via two 4-pol speaker twist connectors. The loudspeaker enclosure shall be trapezoidal in shape. It shall be constructed of 1/2-in thickness Baltic birch plywood with textured black coating and it shall be finished in black. The front of the loudspeaker shall be covered with a perforated steel grille. |  |  |  |
| 6 | Sub – bass loudspeaker system. Quantity 4. (Main front speaker systems) The sub-bass loudspeaker system shall incorporate one min 18", 4" voice coil long excursion driver, with a nominal impedance of 8 Ohm. The system shall produce a Sound Pressure Level (SPL) of min 107 dB SPL on axis at 1 meter with a power input of 1 Watt, and shall be capable of producing a peak output of min 137 SPL on axis at 1 meter. Frequency response shall be min 35Hz to max 120kHz (±3dB). The system shall include an active crossover and amplified controller: 4x1 in parallel on 2 bridged channels, and 4x4 on 1 channel. Crossover frequencies: 60 Hz – 85 Hz - 120 Hz. Electrical connections shall be made via on of two 4-pol speaker twist connectors. It shall be constructed of 1/2-in thickness Baltic birch plywood with textured black coating and it shall be finished in black. The front of the sub bass shall be covered with a perforated steel grille. |  |  |  |
| 7 | 4x1 Powered controller. Quantity 1. (Main front speaker systems) Power amplifier and loudspeaker controller shall have: Number of channels: 4x amplifiers channels, 2 by 2 bridgeable. Max. output power (8Ω) 4 x 600W Max. output power (4Ω) 4 x 900W Max. output power (2Ω) 4 x 1300W Audio Inputs: 4x balanced analogue inputs on XLR3 with a second XLR3 for linking, using 24bit converters and 4x digital inputs via the optional network card slot at the back. Power Outputs: 4x speaker twist connectors outputs using internal power relay for automatic output assignment regarding setup. Analog Inputs channels: 4x channels, analogue inputs on XLR 3 with a second XLR 3 for linking. Slots: for networking audio cards, 4x digital audio inputs and remote control. Front panel with LED indicators. |  |  |  |
| 8 | 4x4 Powered controller. Quantity 1. (Main front speaker systems) Power amplifier and loudspeaker controller shall have: Number of channels: 4x amplifiers channels, 2 by 2 bridgeable. Max. output power (8Ω): 4 x 1900W Max. output power (4Ω): 4 x 3300W Max. output power (2Ω): 4 x 4000W Audio Inputs: 4x balanced analogue inputs on XLR3 with a second XLR3 for linking, using 24bit converters and 4x digital inputs via the optional network card slot at the back. Power Outputs: 4x speaker twist connectors outputs using internal power relay for automatic output assignment regarding setup. Analog Inputs channels: 4x channels, analogue inputs on XLR 3 with a second XLR 3 for linking. Slots: for networking audio cards, 4x digital audio inputs and remote control. Front panel with LED indicators. |  |  |  |
| 9 | Wall support. Quantity 3. (Main front speaker systems) Wall support for loudspeakers shall be used to wall mount speaker cabinet that has a standard hole (top hat) in its base. It shall be compatible with trapezoidal in shape loudspeakers. Wall support shall provide min. 3 mounting positions. It shall be supplied with mounting fixings. Heavy-duty fixings shall be used to secure the bracket to the wall. Min. three full-range loudspeakers units shall be held up by the wall support. Black finished heavy all steel construction. |  |  |  |
| 10 | Fix bumper. Quantity 3. (Main front speaker systems) Fix bumper (fixed holder for loudspeakers) shall be used for 3-cabinet (max) applications. It shall be compatible with trapezoidal in shape loudspeakers. It shall be supplied with mounting fixings. Heavy-duty fixings shall be used to secure the bracket to the wall. Black finished heavy all steel construction. |  |  |  |
| 11 | Set of 12mm screws. Quantity 9. (Main front speaker systems) Set of 12mm screws shall be compatible with above mentioned wall support. Screws shall be made of stainless steel. |  |  |  |
| 12 | Flight case 16U. Quantity 1. (Main front speaker systems) Type: Removable front and rear. Material: min 7 mm plywood multilayered glued, black laminated, with 1" layer of shock-absorbing foam between rails and outer wall. Rack shall be with heavy duty fittings, metal railing/ball corners, aluminum profile frames with rounded edges, spring loaded carry handles and wheels. Also, the rack shall be mounted with handles for transport, rails and screws for installation of rack components. Rack shall strip front and rear. All washers + nuts + screws shall be included |  |  |  |
| 13 | The 2-way loudspeaker system. Quantity 6. (Side-ways and rear speaker system) The two-way full range loudspeaker systems shall incorporate an 8" cone transducers and a 1" exit compression drive on dispersion horn, with a nominal impedance of 8 Ohm. The system's horizontal coverage shall range from min 50º to min100º, with vertical coverage of min 55º. The horn shall be rotating in 4 directions. The system shall produce a Sound Pressure Level (SPL) of min 98 dB SPL on axis at 1 meter with a power input of 1 Watt, and shall be capable of producing a peak output of min 132 SPL on axis at 1 meter. Frequency response shall be min 60Hz to max 20kHz (±3dB). Electrical connections shall be made via two 4-pol speaker twist connectors. The loudspeaker enclosure shall be trapezoidal in shape. It shall be constructed of 1/2-in thickness Baltic birch plywood with textured black coating and it shall be finished in black. The front of the loudspeaker shall be covered with a perforated steel grille. |  |  |  |
| 14 | Wall support. Quantity 6. (Side-ways and rear speaker system) Wall support for loudspeakers shall be used to wall mount above mentioned the two-way full range loudspeaker systems. It shall be compatible with trapezoidal in shape loudspeakers. Wall support shall provide horizontal position for loudspeakers. It shall be supplied with mounting fixings. Heavy-duty fixings shall be used to secure the bracket to the wall. Black finished heavy all steel construction. |  |  |  |
| 15 | Controller device. Quantity 3. (Side-ways and rear speaker system) An advanced analogue processor shall configure from a combination of min two electronically balanced analogue inputs and min three outputs with each output having facilities for crossover, parametric EQ, mid-filter limiters, and delays, in a single rack-mountable unit. The controller device shall provide dynamic, voice coil temperature and diaphragm displacement protection in order to prevent transient audio peaks from causing distortion further up the audio chain or damaging loudspeaker components. Maximum input shall be 22dBu. Total harmonic distortion: less than 0.1%. The device shall have clear front panel LED indication. The chassis shall be constructed of aluminum profile + sheet metal material, suitable for the peripheral devices. Input power shall be 110-220VAC, 50-60Hz with a rear panel earth-lift. |  |  |  |
| 16 | Power amplifier. Quantity 3. (Side-ways and rear speaker system) Power amplifier with: Output power, 1kHz: 750W on 8ohm,1 100W on 4ohm, 2200W on 2ohm. Cooling: 2x variable-speed fan. Power band width: 10 Hz – 40 kHz Total harmonic distortion: less than 0.1% Frequency response: ±0.5 dB, f = 20 Hz – 50 kH Power amplifier shall be equipped with independent sweepable high-pass and low-pass filters on each channel. Also, equipped with speaker twist connectors jacks, binding post, and 1/4-inch output jacks for each channel, balanced XLR and 1/4-inch TRS jacks for channel input. Amplifier shall be in a single rack-mountable unit, constructed of aluminum profile + sheet metal material, suitable for the peripheral devices. |  |  |  |
| 17 | Flight case 16 U. Quantity 1. (Side-ways and rear speaker system) Type: Removable front and rear. Material: min 7 mm plywood multilayered glued, black laminated, with 1" layer of shock-absorbing foam between rails and outer wall. Rack shall be with heavy duty fittings, metal railing/ball corners, aluminum profile frames with rounded edges, spring loaded carry handles and wheels. Also, the rack shall be mounted with handles for transport, rails and screws for installation of rack components. Rack shall strip front and rear. All washers + nuts + screws shall be included. |  |  |  |
| 18 | Mixing console for larger applications. Quantity 1. The mixer shall be a desktop digital mixer with 33 motorized faders (32 channels + 1 master). Main characterisitcs: - 48 input mixing channels (40 mono + 2 stereo + 2 return) - 20 Aux (8 mono + 6 stereo) + Stereo + Sub buses - 32 analog XLR/TRS combo mic/line inputs + 2 analog RCA pin stereo line inputs - 16 analog XLR outputs - 1 expansion slot - 4 custom assign knobs The mixer shall include 34 x 34 digital recording/playback channels via USB 2.0 + 2 x 2 via a USB storage device. A separate section of 8 motorized faders shall control DCA groups with roll-out. Mixing console shall have min 7-inch colour touchscreen with a high luminance backlight on the mixing surface. The fader and rotary controls shall be of a high contrast colour to the mixer surface. The mixer shall have an optimised fan-less airflow design for silent operation. |  |  |  |
| 19 | Speaker cables. Quantity 1. Length 200m. Professional speaker cable shall consist of 2 cores. Materilal for isolation and overall sheath shall be hydrophobic polymeric material. Conductor area: 2.50 mm²; Conductor construction: 50 x 0.25mm bare copper; Conductor resistance: 8 Ohm/km; Diameter 8 ± 0.3 mm; Speaker cable shall be halogen-free and flame-retardant. Colour: grey or black. |  |  |  |
| 20 | Signal cables. Quantity 1. Length 400m. Professional signal (microfone) cable shall consist of 2 twinned cores. Materilal for isolation and overall sheath shall be hydrophobic polymeric material. Conductor Area 0.22 mm2 Conductor construction: 7 x 0.20mm bare copper; Conductor eesistance: 88 Ohm/km; Diameter 4.2 ± 0.2 mm; Speaker cable shall be halogen-free and flame-retardant. Colour: grey or black. |  |  |  |
| 21 | Procesor for cinema. Quantity 1. Multichannel digital cinema audio processor with the following basic characteristics: - Integrated Equalizer Parameter; - Raer Subwoofer; - Optional BLU link; - Inputs 8 channels AES / EBU, 1 coax (S / PDIF) and one optical audio cable with optional decoding; - Sound / video synchronization capability for each format (0-190ms) and surround delay (0-150ms). |  |  |  |
| 22 | LED PAR64 (Parabolic Aluminized Reflector). Quantity 16. Parabolic Aluminized Reflector shall be used for stage lighting. It shall be control via DMX switches. Each PAR shall have min 36W high power LEDS (min 36x1W with Red, Green and Blue) with separate optic lenses. LED PAR 64 shall be completely silent – without fan; Number of DMX channels: 6 DMX Channels; 4 modes: DMX, Sound to light, Color Macro and manual Each PAR shall include hanging bracket with mounting hardware which shall be made of quality cast aluminum shell. Colour: black. |  |  |  |
| 23 | LED fresnel with barndoor. Quantity 6. LED fresnel with barndoor shall be used for stage lighting. Light Source: min 16 LEDs (WW) 10 W Motorized zoom: min 25 to 76° Beam Angle: min 14° to 55° Field Angle: min 25° to 76° Zoom Angle: min 25° to 76° Illuminance (25°): min 1,860 lux @ 5 m; Illuminance (76°): min 285 lux @ 5 m; Control Protocol shall be dimmer, DMX. Electrical connections shall be made via speaker twist connectors. LED fresnel shall include hanging bracket with mounting hardware which shall be made of quality cast aluminum shell. Colour: black. |  |  |  |
| 24 | Scan LED. Quantity 4. LED fresnel with flat mirrored scanner shall be with red, green, blue and white beams. Min. 52 total LEDs: 16 red, 12 green, 12 blue and 12 white. Beam angle: 21°. It shall be control via DMX switches. It shall have LCD DMX function display (4 modes: DMX, Master, Slave and Sound) and strobe function. Linkable via XLR connections. Scan LED shall include hanging bracket with mounting hardware which shall be made of quality cast aluminum shell. Colour: black. |  |  |  |
| 25 | Lap top. Quantity 1. Lap top shall perform for light system on the scene. Capacity of hard disk shall be min. 500 GB. Size of the RAM shall be min 4 GB. Processor with min: 2.00 GHz, 3 MB cash memory, 2 cores. Monitor resolution of 1024x768. USB 2.0 port for DMX interface, Hardware with min 500 MB drive space available. Lap top shall be with lighting controller software and hardwer for programming color-mixing fixtures. Lap top needs to be equipped with all external devices (accessories) to ensure full functionality of the system. |  |  |  |
| 26 | Speaker cables. Quantity 1. Length 200m. Professional speaker cable shall consist of 2 cores. Materilal for isolation and overall sheath shall be hydrophobic polymeric material. Conductor area: 2.50 mm²; Conductor construction: 50 x 0.25mm bare copper; Conductor resistance: 8 Ohm/km; Diameter 8 ± 0.3 mm; Speaker cable shall be halogen-free and flame-retardant. Colour: grey or black. |  |  |  |
| 27 | Signal cables. Quantity 1. Length 400m. Professional signal (microfone) cable shall consist of 2 twinned cores. Materilal for isolation and overall sheath shall be hydrophobic polymeric material. Conductor Area 0.22 mm²; Conductor construction: 7 x 0.20mm bare copper; Conductor eesistance: 88 Ohm/km; Diameter 4.2 ± 0.2 mm; Speaker cable shall be halogen-free and flame-retardant. Colour: grey or black. |  |  |  |
| 28 | Display System 3 LCD system. Quantity 1. Digital projector shall be used for presentation and multimedia with 3LCD technology for medium-sized auditoria. Size of effective display area: min. 0.95”; Aspect ratio: 16:10 (supports 4:3, 16:9, 5:4); Graphics display resolution: min 1920 x 1200 pixels (WUXGA); Lumens: min. 7000 lm; Lamp type: laser diode; Screen size(projected distance) : min. 40” to 600”; Contrast ratio (full white / full black) 8000:1; Remote commander shall be supplied. Projector needs to be equipped with all neccesary external devices (accessories) to ensure full functionality of the system. |  |  |  |
| 29 | Screen intended for medium-sized auditoria. Quantity 1. Screen shall perform for Cinema – 35mm film, 2D digital, and 3D digital. It shall offer high contrast and bright pictures. Material: PVC with pearlescent coating; Screen shall be in either perforated or non perforated forms. Thickness 0.3mm; Maximum size: 33 x 11m; Colour: white. |  |  |  |
| 30 | System instalation and commissioning. Quantity 1. All above mentioned positions shall be equipped with all neccesary external devices (accessories) to ensure full functionality of the system. |  |  |  |
| 31 | Training Staff needs to be trained for all installed equipment. It is necessary to provide a minimum two-day training |  |  |  |