|  | Interreg – IPA CBC Bulgaria – Serbia Programme CCI Number 2014TC16I5CB007 |  |
| --- | --- | --- |

Project No 2014TC16I5CB007-2015-1-124: Conservation, revitalization and exhibition of Early Christian basilica and medieval fortress in cross border region

# VOLUME 3

# TECHNICAL SPECIFICATIONS

Contract title:	Restoration, protection and better exposing of roman complex Castra ad Montenesium

Project No	2014TC16I5CB007-2015-1-124

Name:	Conservation, revitalization and exhibition of Early Christian basilica and medieval fortress in cross border region;

Priority Axis 	1. Sustainable Tourism

Specific Objective	1.1 Tourist Attractiveness

Type of Project:	Investment financed by the European Union, in accordance with the rules of Interreg – IPA CBC Bulgaria – Serbia Programme CCI Number 2014TC16I5CB007

Location:	Municipality of Montana, Republic of Bulgaria

SECTION 1: PARTICULAR TECHNICAL SPECIFICATION

## 1. Background and purpose of the Project

The project Restoration, protection and better exposing of roman complex Castra ad Montenesium in Montana includes

- Restoration activities of the located at the entrance of the fortress early Christian temple – basilica-cleaning ruins and the surrounding area from vegetation and land masses, levelling of terrain, development of drainage system for premises and surrounding terrain calculated for heavy rains, retaining the original walls “in situ”, building up the walls for about 1 m with the materials of the same type and look as the original ones in order to give the visitors an impression of the construction techniques of the temple.

- Activities allowing the visitors to pass through the renovated basilica and connecting it to the already restored part of the complex Castra ad Montenesium.

- Installation of lighting and video surveillance systems to protect the complex from vandalism and looters harmful activities;

- Reforestation and landscaping activities

The aim of the project is development of tourist attraction Castra ad Montenesium – renovation, conservation and exposition of early Christian basilica and protection of fortress Castra ad Montenesium.

2. Current situation

The site is located in the north-western corner of the city. It is at the foot of the hill on top of which are ruined walls of a Roman fortress in the IV-V century AD. They are partially preserved, restored and exhibited. Early Christian basilica has been researched archaeologically in the 80s of the twentieth century by G. Alexandrov in archaeological season of 2010.

The excavations of the site were made by Assoc. Prof. D. Kabakchieva. In subsequent years at the site are not implemented any activities - archaeological studies have not been completed. To date, discovered in 2010 walls are not readable at the terrain overgrown with low vegetation - grass and shrubs. The site is not available for visitors.

The southwest wall of the basilica is dug into the steep north-eastern slope of the hill. From the northeast side of the hill and basilica is Izvora Street. On the south side of the ruins very close passes pedestrian alley leading to the southern entrance of the fortress "Kale". However, the object is located close to the street and walkway; it is difficult to access by pedestrians and for vehicles.

3. Scope and description of the construction works

The scope of works covers Restoration, protection and better exposing of roman complex Castra ad Montenesium in Montana. The construction works will take place on the municipal property and in general provides the following activities to be implemented in accordance with the approved detailed works design:

Conservation and restoration works should be preceded by:

- Cleaning of the ruins and the surrounding area from vegetation and land masses. Levelling of the surrounding terrain 20 cm under the original level.

- Development of drainage system for premises and surrounding terrain calculated for heavy rains.

- Final archaeological studies for the whole site

3.1. Part Architecture

3.1.1. Conservation activities:

- Retaining the original walls "in situ" / place / should be made after carefully cleaning them from dust, eroded solution and unstable construction material - stone.

- Determination of the original floor levels in the basilica.

- Cleaning of detected original walls of eroded solution in depth with hook and brush.

- Removing the structure of the wall of unstable and eroded stones.

- Cleaning of the beds and preparation for incorporation of same or similar in size and color of the stone mortar solution at deep joint.

- Joint again the original face masonry mortar solution, using the previously prepared "trunks" with widths correlated to the width of the joints and length about 40 cm. By them is incorporated the soldering solution in the depth of the joint. This deep joint remains at about 4 to 5 cm from the face of the wall. The solution that works is a little stronger than classic solution that is used for so called “Face joints”. "The latter is performed" at wet ", ie immediately after making deep grouting.

The front facade joints finish with edges of the grouting stones – does not stand or sink between them therefore it is uneven. Stained with solution while working stones should be cleaned of the solution as soon as possible.

3.1.2. Restoration activities

Performed after the strengthening of the original in the way described above.

- On the renovated walls should be applied mortar solution

- Using a batten thickness 4.00 cm and high 6.00 cm should be formed deep gaps in the face of the wall. They should remain unfilled to mark the border between the original and modern superstructure.

- Over this wide gap should be built wall with imitative similar in type, size, colours and stones to invoice design height. It should be implemented also on an equal joint with the mortar solution.

- At a height of 100 cm from level "finished floor" will runs first "opus" of 4 lines "Roman" bricks with width 4.5 cm, and solution 43/43 cm, cemented with 2 cm mortar. The second "opus" will be after 90 cm. The stone construction will also be of 4 rows of brick mortar.

- The conservation above wall should end with a "hat" with symmetric slant to the faces of the wall so in that way should be brought the ambient water evenly over the entire length of face walls

- For the hats should be used flat river smooth stones sunk to bed in the middle of a hydrophobic mortar. / A sample of the kind of "cap" on the spot should be made in the presence of the designer. /

- The position of the height of conservation - expositional above wall is approximate. The right place should be determined jointly by the archaeologist during performance and writing down in the order book.

- As the floor of the Basilica remains under the open sky instead mortar bricks must be laid on a sand cushion with width 15 cm. The sand should be spread and levelled by tamping on water-permeable geotextile. Under the geotextile should be level 20 cm layer of small stone fraction.

On the tamping sand bag should be arranged in straight rows bricks "Roman" type by using remote frogs with a width of 1 cm. Compulsory at the end of each working day, on the arranged during the day bricks on the floor should be spread sand to fill gaps between the bricks. Then are compacted with a rubber rammer to compact the sand in gaps between the bricks. The aim is to drain the rain water into the joints between the bricks and drained through the floor to leave the object in natural ways, and not to focus on one point of the drain field.

- The original pavement in front of Rome's Basilica should be filled in a similar close to the original manner way with imitative material.

3.2. Part Constructive

The object of this part is a manway over the ancient walls. The manway should be with dimensions 58 m length and 4 m on average transverse designed with metal frame. In the constructive attitude it should be a metal tape of longitudinal combined cross beams and closed profile (in electro-box 120/60/4) and ribs of square box 60/60/4.

Anchoring to the walls should be realized through concrete pads (”pillows”) as the following order of operation:

- carefully should be removed at least one of the ancient stone wall (maybe more than one to achieve min. Dimensions);

- the place formed by a partly demolished wall should be cleaned of dirt, old solution (possibly clay) and the like similar!;

- the shuttering should be one sided (from inside);

- the provided an anchor block should be installed horizontally (do not follow the level but the top end antique wall);

To the footbridges there should be installed a protective parapet height 120 cm. It should be made of thin-walled closed profiles - rectangular 60/30/3 to the handle and square - 30/30/3 and 40/40/3 for belts and vertical stuffing.

The construction, including the rail and should be divided into two expansion joints

This solution allows the installation of all the components to be made of welding as the lengths of the corner seams are along the line of contact at cathetus 4 mm.

Metal structure:

Cold cranked closed (hollow) profiles BS EN 10219-1, Steel S

235 JR H Reh = 235 Mpa; Rm = 360 Mpa

Concrete:

- class B20 (C16 / 20) - unreinforced concrete pads!

- In parentheses are the names of the brands concrete in Eurocode 2.

- Structural analysis was conducted for the following legal cargo:

- useful load (a crowd of visitors) - 400 kg / sq. M

When sizing are used the following regulations:

Ordinance on the basics of designing the structure of construction and migration, degassing on them- 2004;

Eurocode 1 (BDS EN1991-1-1: 2002);

Norms for design of reinforced concrete structures;

Eurocode 2 (EN 1992-1-1 BS: 2004);

Standards for the design of steel structures;
Eurocode 3 (BS EN 1993 -1 -1: 2005

3.3. Part Landscaping

GARDENING, as an integral part of the project for the conservation, restoration and exhibition of early Christian basilica and the surrounding area in regulated plot of land (RPL) in Montana.

Landscaping is a complex of landscape, developing, agricultural and ameliorative measures that should be implemented the first three years after completion of the project, subsequently - in the maintenance of green spaces in the area.

This project provides eradicate of 9-year seedlings by Robinia pseudoacacia (acacia) - 35 pcs., 10 pcs., 11-year Ailanthus glandulosa (Ailanthus), cleaning of dry branches and bushes climbing on the continuing ornamental plants, grass and planting shrubs

3.3.1. Grassing

After analysing the environmental conditions and characteristics of the object it is necessary 300 m2 to be grassed with turf, and the rest – by Sowing grass mixtures in - majority and a small area in pre-positioned geo cells which are filled and retain the soil and grass seed, and then to grass the sod. Due to the complex and unfavourable soil terms is available grassing will be performed with higher rates of grass seed - 300 kg/ha, by choosing the most easy-going conditions of the environment depending on the available on the market and grass mixtures such as:

Decorative twitch (Cynodon productstylon) or Grassblend BREZINA 2 Company Svemar, which fully meets the requirements of the habitat and its role to consolidate the soil and aesthetic object:

| BREZINA 2 - for places along roads, against erosion with field flowers and herbs | Variety | % of weight | % of the grass in the mixture |
| --- | --- | --- | --- |
| 1. Festuca ovina ssp. duriuscula | SPARTAN | 15 | 15 |
| 2. Festuca rubra rubra | PERNILLE | 39 | 30 |
| 3. Festuca arundinacea | MUSTANG | 10 | 12 |
| 4. Dactylis glomerata | FALA | 9 | 15 |
| 5. Lolium perene | SCORE | 15 | 15 |
| 6. Poa pratensis | BALIN | 7 | 8 |
| 7. Lotus cornicolatus | ROCCO | 2 | 3 |
| 8. Leucantheum vulgare | - | 0,4 | 0,4 |
| 9. Cichorium intybus | - | 0,4 | 0,4 |

Technological order for implementation of activities under development of grass areas

I year (from autumn to autumn)

- Levelling with a rake and removal of unusual for soil substrate inclusions.

- Adding fertilizer / phosphorus and part of nitrogen /

- Seeding of grass mixture, planting turf and trampling with hand roller

- Watering after grassing

- Spreading the remaining amount of fertilizer (in the spring and early summer) with subsequent watering

- Mowing

II and III year

- Spreading of phosphorus fertilizer (maybe in years)

- Nitrogen fertilization - April-May

- Fertilizing - May-June

- Raking (levelling vertically) - removing dead and dry grass basis through the grass rake. Later on, due to frequent mowing and natural waste on the soil is formed a thin film of dry grass. This stops the access to the ground water and food substances hampers air exchange and hinder the growth of grass.

- The dried grass should not be removed by brushing and it should only be spread to aerated grass array. Later on the grass will be decomposed and enriched with Soil organic matter.

- Mowing - more frequent mowing without removal of the mowed grass supports the grass array and enriches the soil with organic matter. If used, however, grass mixtures BREZINA 2, mowing can be saved. Planting shrubs is to strengthen the slope and achieve decorative effect.

3.3.2. Landscaping by planting decorative trees and shrubs

Existing vegetation of the site landscaping should be planted in time with the following composition: Robinia pseudoacacia L. (acacia), Acer platanoides L. (six), Acer negundo (Acer negundo). Herbaceous dominated by grasses, among which the most frequently dominate beard (Chrysopogon gryllus Trin.) And Belizmata (Dichantium ischaemum Roberty). In processes on-going succession in degradation of forest communities most of the tree species are coated with Clematis vitalba L. (clematis).

Much of the plant community have arisen under the influence of anthropogenic factor and represent bush arrays with low ecological importance.

Due to the strong distortion of relief from on-going erosion processes denudation at some places of the soil and soil restoring landscapes with valuable plant communities will be difficult and species composition that we can use will have to be resistant to these conditions. Moreover it must naturally fit in landscape and to highlights the archaeological site. The selection of plant species is made to take into account the following environmental conditions: - relief exposure, rainfall, acidity / alkalinity of the poor soil with a lot of rock included in them.

3.3.3. Planting bushes

Specific environmental conditions of the site and its future role as connection between archaeology and urban landscape require special attention as the choice of suitable vegetation and land reclamation and also melioration activities and maintenance. At few places of proposed for landscaping and landscape device field will be planted shrubs - all 75 pcs. These are steep places close to the road and groups in rocky terrain. Planting is carried out in appropriate conditions in the fall before getting colder or better - in the spring.

Planting shrubs is done manually, while transportation, storage and spreading them need necessary measures to avoid their drying and damage. If necessary, the root system can be trimmed. Planting bushes in holes with dimensions 30/30/30 cm distance 0.70 m, staggered in accordance with the submitted drawing.

Usually are used standard for planting saplings.

Since grassing is tightly in designated areas – through sod or seeding, in this part we also plant and the geranium - Geranium macrorrhizum, although not a bush. It is planted in groups at the distance 20 - 25 cm from each other and holes 15/15/15 cm provided for planting plants are 60 pcs.

3.3.4. Technological schemes for planting and melioration

I year (autumn to autumn)

- Preparation in autumn - entrenchment of places where will be planted bushes (incl. and geranium)

- mark and plant the saplings and geraniums (in autumn or in the spring)

- In spring - mainly fertilizing with ammonium nitrate and feeding with all fertilizers in the scheme shown in grassing

- Scooping the bushes and geraniums - 3 times

- Additional fertilization with superphosphate and potassium fertilizer feeding / in Autumn /

Year II (autumn to autumn)

- Filling places of demolished trees and geraniums

- fertilizing (with ammonium nitrate) and feeding of ammonium nitrate and potassium fertilizer /in spring /

- Scooping - twice during the growing season

- fertilizing with potassium fertilizer (in the autumn)

Year III

- filling the places of the demolished sapling

- Feeding up with ammonium nitrate / spring /

- Scoping - once during the growing season (if the bushes are do not cover the entire planned area)

- Fertilization standards are the same as in grassing.

3.3.5. Planting care

Planting care for created shrub groups and geranium consist in Scooping and watering. Scooping between the bushes and geranium is done till connecting the groups, ie to tightly cover the soil with vegetative mass Filling in arrays happens when the interception is less than 75 - 60% and replanting - is below 60%. Ameliorative measures are expressed in fertilization and watering after fertilization and, if necessary during the vegetating season.

3.4. PART ELECTRO-TECHNICAL

3.4.1. Installation of lighting system

- Delivery and installation of stairs 8.2 m

- Delivery and installation belaying cleats - 0.25 m

- Delivery lighting body with 100W metal halide lamp

- Delivery and installation 6W 24V LED projector

- Making excavation and burial

- Laying pipe d = 50

- Supply and download cable СВТ 4 x 2.5

- Supply and download cable СВТ 3 x 2.5

- Supply and download cable СВТ 2 x 4.0

- Placing warning tape

- Making shafts 400/400/300

- Installation terminal boxes

- Piling call for grounding

- Delivery and installation Switchboards

3.4.2. VIDEO CONTROL

Installation of video surveillance system

- 2 MP digital cameras

- Stands for cameras

- DVR 16 channel digital HD DVR

- HDD 4TB

- Cable RG58

- Cable ППВР 2 х 0.75

- Cable СВТ 2 х 1.5

- Cable СВТ 3 х 2.5

- Cable FTP CAT. 6

- router 150 MB

- PATCH Cable

- UPS 1300W

- couplers

- supply 12V 150W

- power connectors WAGO

- Jack for external mounting CAT.6

- BNC coupling

- power receptacle 5.5/2.1

- junction boxes IP65

- Outdoor cabinet with cooling and heating

The implementation of the contract shall be executed with full compliance with the description of the work design which is an integral part of tender documentation. The work design includes explanatory notes, bill of quantities and design drawings.

- The maximum period of implementation of construction works should be 9 months from the date of the giving access to the Site.

SECTION 2: GENERAL REQUIREMENTS

The Technical Specification is an integral part of the Tender Documentation /TD/ together with the provisions of the Contract, approved Investment design, the detailed design drawings, and the other contract documents. The Specification specifies and further develops the requirements for the implementation of the construction works under the Contract. Implementation of construction and assembly work must be consistent with all relevant execution of specific types of works legal and regulatory framework, technical rules and regulations and applicable standards following sequence and technology of the performance of different types of works in different parts of the site.

1.	STANDARDS

All materials and equipment shall meet the following technical specifications:

- Bulgarian standards transposing European or international standards, or equivalent;

- European technical approvals (with or without guidance);

If the technical specifications under item 1 or 2 do not exist, building materials should meet recognized national technical specifications.

Where the technical specifications cannot be defined in the top row, including where they do not exist, have not been published or are not enforced, they are determined by:

- Bulgarian standards transposing European or international standards, or equivalent;

- Bulgarian standards or equivalent;

In case that there are no published standards apply Bulgarian technical approvals and regulations for the design, implementation and supervision of works or individual construction works.

The materials which are used must correspond to the requirements of the standardized documents asset in Bulgarian State Standards Act.

If the Contractor should wish to supply material or execute work to an alternative national standard or international specification he shall give full details of his proposal in writing to the Supervisor.

2.	CONTROL OF WORKS

The Contracting Authority will provide a consultant who will supervise construction with investment functions, according to Art. 166, par. 1, item 1 of the Bulgarian Law on Spatial Planning. The Contracting Authority and / or supervisor may at any time inspect the work, control technology performance and issue instructions to remove the defects, according to the specified technology and method of implementation. If found serious defects, errors and low quality performance, the Contracting Authority shall notify the Contractor that breached the contract and should stop to work. The Contractor shall always provide access to the construction site of the authorized representatives of the Contracting Authority and the Supervisor.

2.1	Contractor's equipment

The Contractor shall furnish equipment which will be efficient and appropriate to secure a satisfactory quality of work and a rate of progress which will insure the completion of the Works within the time stipulated in the Tender. If at any time such equipment appears to be inefficient, inappropriate or insufficient for securing the quality of work required or for the rate of progress, the Supervisor may be entitled to order the Contractor to increase the efficiency, change the character or hire additional equipment, and the Contractor shall conform to such order.

2.2 Protection of existing structures and utilities

The Contractor shall assume full responsibility for the protection of all buildings, structures and roads existing in the area of the construction site, public or private, whether or not they are shown on the drawings.

When starting the contract to be invited all companies and institutions exploiting underground lines and facilities, specifying the exact location of existing underground lines/facilities and in their vicinity excavation should be carried out very carefully and in places where necessary, manually.

The Contractor has to pay special attention to avoid any damages on any protected areas if any.

Any damage resulting from the Contractor's operations shall be repaired at his expense.

2.3	Requirements regarding the provision of safe and healthy working conditions. Plan for Safety and Health

During the execution of construction works Contractor shall comply with the requirements of Ordinance № 2 of 2004 on minimum requirements for health and safety in the course of construction works, as well as all other applicable regulations and standards for safety and hygiene, industrial and fire safety during construction and operation of such sites, and also to ensure the safety of all persons who are on site.

The Contractor shall comply with the approved by the Contracting Authority and the competent authority a plan for safety and health for construction.

2.4	Approval of sources, materials and plants

Materials and plants incorporated must originate in any eligible source country as defined in of IPA Cross-border programme. For these purposes, ‘origin’ means the place where the goods are mined, grown, produced or manufactured and/or from which services are provided. The origin of the goods must be determined according to the EU Customs Code or the applicable international agreement. Material that originates from sources, which are not approved by the Supervisor, cannot be used for Works.

Approval of a source does not mean that all material in the source is approved. The Contractor has to ascertain by continuous control check measurements that only material which complies with the requirements specified in the various clauses of these specifications will be used for the Works.

2.5	 Requirements regarding environmental protection

In the execution of construction works contractor must confine its action within the construction site.

After completion of construction works the Contractor shall reimburse the construction site by the project for development and park arrangement - to withdraw all their machinery and unused materials and leave the site clean of debris and completely finished.

The Contractor shall clear way and remove from the site any wreckage, rubbish and temporary works, which are no longer required.

2.8	Responsibility of the Contractor

Approvals from the Supervisor do not relieve the Contractor from his obligations or responsibilities under the Contract.

3.	ADMINISTRATIVE SPECIFICATIONS

3.1. Implementation Programme

Implementation programme for delivery of works shall be prepared consistent with the duration of the project’s implementation and the works activities implementation. The programme shall be presented in a format which includes a bar chart of all the major activities. No later than five working days after the end of each month, the Contractor shall submit an updated programme to the Supervisor. This shall show, as a minimum:

- progress made during the past month, compared with planned progress

- activities planned for the next month.

3.2. Progress meetings

The Contractor shall agree with the Supervisor and the Contracting Authority for dates for regular progress meetings. These meetings shall normally be held monthly, no later than 10 working days after the completion of each month.

3.3. Quality assurance

The Contractor shall institute a quality assurance system to demonstrate compliance with the requirements of this contract. The system shall be in accordance with the details stated in the Contract. The Supervisor shall be entitled to audit any aspects of the system.

4.	IMPLEMENTATION OF THE WORKS

4.1.	Materials

All Materials supplied to perform the Permanent Works under the contract shall be new products. Second-hand Materials will not be accepted.

Supply of all materials and equipment required for the execution of construction - assembly work according to the approved investment project is the responsibility of the Contractor. Any change to the approved design must be coordinated and approved the Contracting Authority.

All materials that would be used in the construction must be accompanied by relevant certificates of origin and quality, instructions for use and a statement certifying the compliance of each of the nested construction products with the essential requirements for building works, in accordance with the Law on technical requirements for products and by-laws to it.

The Tender drawings constitute the drawings issued for construction/ installation/ execution.

4.2.	Testing

Reliable shall be only the type and amount of tests performed in conformity with the prescription of this Technical specification, except when this right is granted to the Contracting Authority.

Tests and the measurements of the performed construction works should be carried out by certified laboratories and shall be certified by protocols. The Contracting Authority may require additional tests when the results obtained are uncertain.

Beside the tests specified in this Specification, the Contracting Authority may require additional tests to establish possible hidden omissions and effects. Costs for these tests shall be entirely at the Contractor’s expense if such defects are confirmed.

4.3.	Inspection and measurement of works

In the process of construction and assembly works must be compose all the necessary documents and records provided for in Ordinance № 3 from 31.07.2003 for preparation of documents and reports during construction.

The Contracting Authority may at any time inspect the quality and measure the amount of works performed. If this cannot be done with the Contractor’s assistance, a deadline shall be fixed for hiring external specialists. In this case, the expenses shall be paid by the Contractor.

5.	PREPARATORY WORKS

Before starting the works the Contractor has to perform some preparatory works at the site.

5.1.	Boards and signs

The Contractor shall mount and maintain in good condition a board with the name of the project and the financing institution.

5.2.	Setting out the site

The Contractor shall in co-operation with the Contracting Authority set out the total site to be used for construction.

5.3.	Temporary facilities

The Contractor shall effects all expenditure for establishing, operation and removal of temporary facilities if such are needed for the good performance of the Contract. All needs for establishing such facility shall be duly justified.

5.4.	General supply facilities

- Sanitary Arrangements

The Contractor shall provide for and maintain temporary sanitary facilities on the site for the use of all persons connected with the Works. The Contractor shall keep the facilities in a clean and sanitary condition, and shall post notices and take such precautions as may be necessary to keep the site clean.

- Water supply

The Contractor shall provide for and maintain an adequate supply of potable water for his use. The water supply shall be used for construction purposes and for consumption in the temporary facilities.

- Power supply

All electrical power required by the Contractor shall be provided by him at his own expense. The Contractor prior to taking-over of the Works shall remove all temporary installations if it is not agreed upon that the Contracting Authority takes over the installations.

IMPORTANT:

All works must be implemented in accordance with investment design and corresponding standards.

In case that in investment design - explanatory notes, bills of quantities, drawings, specifications, and everywhere in the documentation for tendering are set specific brand/mark, model, type, standard materials and products to be considered that whereas and equivalent.

The Contracting Authority does not require specific make, model, type, standard materials and products that will be in the works as long as you comply with the specifications of the designers and the essential requirements for building works.

Annex 1 – Bill of Quantities

| Bill of Quantities |  |  |  |
| --- | --- | --- | --- |
| No | Description of works | Measure | Quantity |
|  | Part І. Archaeological excavations, restoration, conservation, exhibition and socialization |  |  |
|  | А. Disassembly |  |  |
| 1 | Logging and clearing of individual trees and bushes, incl. loading, transportation and unloading | m2 | 280.00 |
| 2 | Removing the humus manually up to a thickness 10 cm | m2 | 280.00 |
| 3 | Transferring earth soils to 3 m horizontally or 2 m vertical distance – Kshattered = 1.3 | m3 | 42.50 |
| 4 | Manually loading, transport up to 50 m and unloading of earth soil with wheelbarrows | m3 | 42.50 |
| 5 | Loading of earth soils truck - manual | m3 | 42.50 |
| 6 | Transportation ground soil by truck 10 km | m3 | 42.50 |
| 7 | Unloading on earth soils truck - manual | m3 | 42.50 |
| 8 | Cleaning of the original walls of weathering and eroded materials | m2 | 148.00 |
| 9 | Dismantling of through overuse the Spoletto - carefully - conservation and sub-assemblies | pcs | 1.00 |
| 10 | Cleaning of the reserved flooring eroded materials - highly broken floor brick and stucco | m2 | 291.00 |
| 11 | Dismantling of crushed stone threshold, with marking elements for secondary assembly, conservation and restoration | pcs | 1.00 |
| 12 | Cleaning of the brick T-shaped columns of powdered brick and vapid solution | m2 | 11.80 |
|  | B. Restoration and conservation |  |  |
| 13 | Strengthening the foundations of the original walls by filling the voids with stone solution | m3 | 36.00 |
| 14 | Laying of the ruined foundations of the walls to step preserved original (floor level) | m3 | 167.00 |
| 15 | Filling in archaeological excavations in the premises with inert materials eroded from the Basilica to the drainage level | m3 | 15.00 |
| 16 | Filling in archaeological excavations in the premises with inert materials eroded from the Basilica to the drainage level | m3 | 19.50 |
| 17 | Preparation of bed and installation of canned Spoleto wall in the same way the original | pcs | 1.00 |
| 18 | Laying of mortar solution on all original crowns the walls | m2 | 28.00 |
| 19 | Construction of joint over all saved original parts of the walls - one-sided on the facades | m | 280.00 |
| 20 | Filling in the missing bricks in the original brick "opus" with imitative equipment and solution | m3 | 4.50 |
| 21 | Shaping the pages of all inputs imitative brick construction to design heights | m3 | 156.00 |
| 22 | Construction of imitative conservation - restoration stone wall above the original preserved to the level of the first brick "opus" and the two "opus" -a all types | m3 | 210.00 |
| 23 | Construction of the second "opus" the entire length of the walls - one-sided | m3 | 76.00 |
| 24 | laying of imitative stone wall above the second "opus", forming a silhouette of the ruins, according to project heights | m3 | 42.00 |
| 25 | Shaping the protection cap of the crown of the walls, acc. technology described by project heights | m2 | 115.00 |
| 26 | Cleaning facades of the walls of mortar and grouting equal joint walls and facades | m2 | 782.00 |
| 27 | Laying protective layer of plastic lime sand solution on preserved "witnesses" of the original flooring | m2 | 25.00 |
| 28 | Laying brick columns to the design height | m3 | 3.50 |
| 29 | Withdrawal of earth from all rooms of the original pavement to drain project level | m3 | 112.40 |
| 30 | On-site installation of the copy of the threshold stone at the main entrance of the basilica | pcs | 1.00 |
| 31 | Construction and putting in place a remove and overuse antique torso served as a stage, a new workable rough stone blocks | pcs | 1.00 |
| 32 | Laying and compaction of stone fraction on the floor of all rooms except those with preserved original surface | m2 | 56.20 |
| 33 | Laying of water-permeable geotextile floors | m2 | 291.00 |
| 34 | Sandy surface with a thickness of 15 cm Including consolidation | m2 | 42.20 |
| 35 | Arrange bricks size 34.34. 4.5 cm floors, modelled on the original wide gap | m2 | 291.00 |
| 36 | Filling and sealing of joints 4.5 cm deep with sand | m2 | 291.00 |
| 37 | Making copies of original artefacts from the Basilica of the exposure and the restored space | pcs | 4.00 |
|  | С. Walls space north and northeast of the basilica. |  |  |
| 38 | Cleaning the original wall of weathered and eroded materials - stone - visible foundations, walls and top of the walls | m2 | 570.00 |
| 39 | Strengthening the original foundations, face and crown the walls by filling the voids with imitative stone size | m2 | 515.00 |
| 40 | Filling the caverns single in front of the original walls with similar size and type of the material stone | m2 | 37.00 |
| 41 | Laying of mortar solution on all original crowns the walls | m2 | 170.00 |
| 42 | Making accentuate gap above all preserved original parts of the walls - one-sided and two-sided according to wall | m2 | 190.00 |
| 43 | Construction of imitative conservation stone superstructure, the entire length of the walls with an average height of 120 cm | m3 | 197.00 |
| 44 | Forming a protective "cap" the crown of the walls according to the described technology performance | m2 | 190.00 |
| 45 | Cleaning the walls of mortar and grouting equal joint walls and facades | m2 | 480.00 |
|  | Part ІІІ. Electro-technical |  |  |
| 1 | Delivery and installation of stairs 8.2 m | pcs | 10.00 |
| 2 | Delivery and installation belaying cleats - 0.25 m | pcs | 10.00 |
| 3 | Delivery lighting body with 100W metal halide lamp | pcs | 10.00 |
| 4 | Delivery and installation 6W 24V LED projector | pcs | 20.00 |
| 5 | Making excavation and burial | m | 260.00 |
| 6 | Laying pipe d = 50 | m | 260.00 |
| 7 | Supply and download cable СВТ 4 x 2.5 | m | 280.00 |
| 8 | Supply and download cable СВТ 3 x 2.5 | m | 42.00 |
| 9 | Supply and download cable СВТ 2 x 4.0 | m | 62.00 |
| 10 | Placing warning tape | kg | 3.60 |
| 11 | Making shafts 400/400/300 | pcs | 60.00 |
| 12 | Installation terminal boxes | pcs | 10.00 |
| 13 | Piling call for grounding | pcs | 10.00 |
| 14 | Delivery and installation Switchboards | pcs | 1.00 |
|  | B. VIDEO CONTROL |  |  |
| 15 | 2 MP digital cameras | pcs | 20.00 |
| 16 | Stands for cameras | pcs | 20.00 |
| 17 | DVR 16 channel digital HD DVR | pcs | 2.00 |
| 18 | HDD 4TB | pcs | 2.00 |
| 19 | Cable RG58 | m | 800.00 |
| 20 | Cable ППВР 2 х 0.75 | m | 600.00 |
| 21 | Cable СВТ 2 х 1.5 | m | 200.00 |
| 22 | Cable СВТ 3 х 2.5 | m | 200.00 |
| 23 | Cable FTP CAT. 6 | m | 200.00 |
| 24 | router 150 MB | pcs | 1.00 |
| 25 | PATCH Cable | m | 2.00 |
| 26 | UPS 1300W | pcs | 1.00 |
| 27 | couplers | pcs | 2.00 |
| 28 | supply 12V 150W | pcs | 2.00 |
| 29 | power connectors WAGO | pcs | 60.00 |
| 30 | Jack for external mounting CAT.6 | pcs | 1.00 |
| 31 | BNC coupling | pcs | 40.00 |
| 32 | power receptacle 5.5/2.1 | pcs | 20.00 |
| 33 | junction boxes IP65 | pcs | 20.00 |
| 34 | Outdoor cabinet with cooling and heating | pcs | 1.00 |
|  | Part ІV. Redevelopment |  |  |
| 1 | Uprooting of trees | pcs | 45.00 |
| 2 | Cleaning the deadwood from trees and climbing plants | pcs | 29.00 |
| 3 | Waste disposal | pcs | 11.00 |
| 4 | Align areas - manual | ha | 0.150 |
| 5 | Cutting lawns | ha | 0.150 |
| 6 | Digging with picks and cleaning stones | ha | 0.150 |
| 7 | Aligning with paddle | ha | 0.150 |
| 8 | Sowing grass seed | ha | 0.150 |
| 9 | Cover the seed sown with paddle | ha | 0.150 |
| 10 | Overcrowding roller of the sown grass seed | ha | 0.150 |
| 11 | Dew a new lawn | ha | 0.150 |
| 12 | Planting grass turf | ha | 0.030 |
| 13 | Watering newly planted grass turf | ha | 0.030 |
| 14 | Planting shrubs | pcs | 75.00 |
| 15 | Watering newly planted shrubs | pcs | 75.00 |
| 16 | Embankment of humus in decorative vases | m3 | 0.10 |
| 17 | Planting perennials vases | pcs | 60.00 |
| 18 | Watering planted vases | pcs | 20.00 |
|  | Part, financed by the Municipality of Montana |  |  |
|  | Part ІІ. Vertical planning |  |  |
|  | А. Inside a fortified alley and ladders |  |  |
| 1 | Removal of humus by hand with a thickness of 10 cm | m2 | 225.00 |
| 2 | Slim trench to 0.5 m by hand in earth soil and transfer 3 m horizontal | m3 | 112.50 |
| 3 | Shifting earth soils to 3 m horizontal or 2 m vertical distance - Kshattered = 1.3 | m3 | 73.10 |
| 4 | Manual load transport up to 50 m and unloading of earth soil with wheelbarrows | m3 | 219.40 |
| 5 | Load of earth soils on truck - manual | m3 | 219.40 |
| 6 | Transportation earth soils by truck 10 km | m3 | 219.40 |
| 7 | Unloading of terrestrial soils truck - manual | m3 | 219.40 |
| 8 | Paving stone fraction 0 - 25 mm with a thickness of 15 cm incl. supply | m3 | 34.00 |
| 9 | Broadcast sand wedge fraction incl. delivery | m3 | 11.25 |
| 10 | Shaping steps from railway sleepers, incl. delivery | pcs | 83.00 |
|  | B. Vertical planning in the basilica |  |  |
| 11 | Removal of humus by hand with a thickness of 10 cm | m2 | 130.00 |
| 12 | Slim trench to 0.5 m by hand in earth soils and transfer 3 m horizontal | m3 | 59.00 |
| 13 | Shifting earth soils to 3 m horizontal or 2 m vertical distance - Kshattered = 1.3 | m3 | 93.60 |
| 14 | Manual load transport up to 50 m and unloading of earth soil with wheelbarrows | m3 | 93.60 |
| 15 | Load of earth soils truck - manual | m3 | 93.60 |
| 16 | Transportation earth soils by truck 10 km | m3 | 93.60 |
| 17 | Unloading of terrestrial soils truck - manual | m3 | 93.60 |
| 18 | Paving stone fraction 0 - 25 mm with a thickness of 15 cm incl. supply | m3 | 19.50 |
| 19 | Broadcast sand wedge fraction incl. delivery | m3 | 6.50 |
|  | C. Fence of ancient fortress |  |  |
| 20 | Fence system with a height of 1 800 mm of zinc panels with polyester coating (PVC), incl. Delivery and installation | m | 810.00 |
|  | Part V. Constructions |  |  |
| 1 | Steel pedestrian footbridges, incl. priming and painting | kg | 8 420.00 |
| 2 | Grid floor of footbridges, incl. priming and painting | m2 | 225.00 |
| 3 | Sampling of the anchor holes in the rock | pcs | 64.00 |
| 4 | Piling of steel anchors | pcs | 64.00 |
| 5 | Concrete B20 countervailing pads | m3 | 2.60 |